NDT = Non-Destructive Testing.

It means testing a structure without damaging or destroying the material.

For your bridge project, NDT is basically the technology that lets you look beyond what a camera can see.

### Common NDT methods for bridges

| Method | What it detects | Your system |
| --- | --- | --- |
| 📷 Computer vision | Cracks, spalling, rust | iPhone camera + AI |
| 📐 LiDAR | Deformation, displacement, geometry | iPhone LiDAR |
| 🌡️ Thermal imaging | Moisture, delamination, temperature anomalies | Thermal camera |
| 🔊 Ultrasonic testing | Internal cracks, voids, discontinuities | Raspberry Pi + transducers |
| 📡 GPR | Rebar, tendons, voids, internal structure | Future add-on |
| 📳 Vibration/IMU | Changes in dynamic behavior | Raspberry Pi IMU |
| 🎤 Acoustic/impact echo | Delamination, voids, concrete condition | Microphone/sensor |

### The important distinction

Imagine a concrete bridge girder:

SURFACE

─────────────────────

│  crack

▼

╲   ╱

╲ ╱

│

──────┼──────────────

│

│   ← hidden void

│      ○

│

│   ← rebar/tendon

═══════════════

─────────────────────

A camera might see the surface crack.

LiDAR might see that the girder has deformed.

But neither necessarily tells you what's happening inside the concrete.

That's where NDT comes in.

### For your concept

I'd frame the system as three layers:

1. Vision — "What does the surface look like?"
 Camera + AI

2. Geometry — "Has the structure moved or deformed?"
 LiDAR + GPS + IMU

3. NDT — "What's happening beneath the surface?"
 Ultrasound + GPR + thermal/acoustic methods

So your pitch could be:

“Vision sees the surface. LiDAR sees movement. NDT sees what the surface can't.”

One caveat: NDT doesn't mean the system automatically determines whether a bridge is safe. It provides measurements and indications that engineers can use for a more detailed assessment.

Today 12:18 PM

best way to detect strucutral issues of asphalt with rasperrby pi and sensors. build out the programms i need, which LLMs are best, cost structure for a prototype, I hvae 3 days for a competition

For a 3-day competition, I would not try to build a Raspberry Pi system that directly determines the deep structural capacity of asphalt. That is too ambitious.

Instead, build a mobile pavement inspection system that combines RGB vision + 3D geometry + thermal + motion, then uses AI to turn those measurements into a road condition map.

The important distinction is:

Surface distress ≠ structural capacity.

You can reliably demonstrate cracks, potholes, rutting, deformation, roughness, and thermal anomalies. Subsurface structural problems would eventually require GPR, deflection testing, or other professional NDT. Research supports combining visual, 3D, and thermal sensing for pavement inspection; LiDAR is particularly useful for measuring geometry such as rutting and pothole depth, while thermal imaging can reveal additional distress information.

# The system I'd build

ROAD INSPECTION VEHICLE

│

┌────────────────┼────────────────┐

↓                ↓                ↓

CAMERA            LiDAR            THERMAL

│                │                │

Cracks          Rutting          Temperature

Potholes        Potholes         anomalies

Raveling        Settlement       moisture/

Patches         Roughness        distress

│                │                │

└────────────────┼────────────────┘

↓

IMU

↓

Motion / vibration

│

↓

┌─────────────────┐

│ SENSOR FUSION AI │

└─────────────────┘

↓

ROAD CONDITION MAP

↓

┌──────────────┼──────────────┐

↓              ↓              ↓

LOCATION       SEVERITY       PRIORITY

## The biggest change I'd make to your original idea

Don't make the Raspberry Pi do everything.

Use:

### Raspberry Pi

Sensor hub + data logger

### iPhone

High-quality camera + LiDAR + GPS + UI

### Laptop/cloud

AI inference + processing + dashboard

That gives you much more capability in three days.

# 1. Your MVP sensors

## A. RGB camera — absolutely essential

This detects:

- longitudinal cracks

- transverse cracks

- alligator/fatigue cracking

- potholes

- patches

- raveling

- edge damage

- lane/surface anomalies

A Raspberry Pi Camera Module 3 is inexpensive and supports autofocus/HDR. Current listings are roughly $30–40. Raspberry Pi Camera Module 3

But if you already have an iPhone, I'd use the iPhone camera instead.

### Why?

Much better image quality.

Your Pi doesn't need to spend its limited processing power doing computer vision.

# 2. LiDAR — your best sensor for rutting/deformation

If someone on your team has an iPhone Pro with LiDAR, use it.

You can generate a point cloud:

Normal pavement

───────────────

Rutted pavement

──────\____/──────

↑

rut

You can calculate:

- pothole depth

- pothole volume

- rut depth

- surface elevation

- road roughness

- localized deformation

- settlement

LiDAR-based pavement research specifically emphasizes its ability to produce metric 3D geometry rather than merely classifying visual appearance.

This is probably your strongest differentiator over a simple "AI crack detector."

# 3. Thermal camera

This is where your Raspberry Pi becomes interesting.

A small MLX90640 provides a 32×24 thermal array and can communicate with Raspberry Pi over I²C.

MLX90640 IR Array Thermal Imaging Camera

$68.14 · RobotShop USA

Current examples are roughly $70–100, although prices vary substantially by vendor. Adafruit MLX90640 IR Thermal Camera Breakout

You aren't going to use it to say:

"The road has a 4.7-inch-deep structural failure."

Instead:

"This region has an abnormal thermal signature relative to surrounding pavement."

Thermal imaging has actually been researched for asphalt distress detection, including cracks and potholes, and multimodal visible + infrared imagery has shown useful performance.

# 4. IMU

Put an accelerometer/gyroscope on the inspection platform.

Something like:

Adafruit MPU-6050 6-DoF Accelerometer and Gyro Sensor

$12.99 · Micro Center

can provide:

- acceleration

- vibration

- pitch

- roll

- motion

You can create a crude road roughness signal:

vehicle acceleration

/\       /\

/  \     /  \

___/    \___/    \____

↑

rough area

You could then correlate:

high vibration + LiDAR deformation

which is more meaningful than either signal alone.

# 5. GPS

You want every observation tied to a location.

Even if you just use the phone's GPS for the competition:

Crack

Lat: 34.xxxx

Lon: -118.xxxx

Pothole

Lat: 34.xxxx

Lon: -118.xxxx

Rut

Lat: 34.xxxx

Lon: -118.xxxx

Then your dashboard becomes a GIS road-condition map.

# What I would NOT build in 3 days

Don't waste your competition time on:

❌ GPR
 ❌ ultrasonic asphalt testing
 ❌ professional deflection testing
 ❌ subsurface tomography
 ❌ custom LiDAR hardware
 ❌ training a giant neural network from scratch
 ❌ trying to determine remaining pavement life

Those are excellent Phase 2/3 technologies.

# The software stack

Here's the actual software architecture I'd use.

DATA COLLECTION

│

┌────────────┼────────────┐

↓            ↓            ↓

Camera        LiDAR       Thermal

│            │            │

└────────────┼────────────┘

↓

Data Fusion

↓

┌──────────────┼──────────────┐

↓              ↓              ↓

Vision Model    Point Cloud      Thermal

│            Analysis       Analysis

↓              ↓              ↓

CRACKS         RUTS          ANOMALIES

POTHOLES       DEPTH

PATCHES        ROUGHNESS

│              │

└──────────────┼──────────────┘

↓

RISK ENGINE

↓

DATABASE / GIS

↓

DASHBOARD

# Program #1 — Data collector

Python.

Have the Pi record:

timestamp

GPS

temperature

accelerometer

gyroscope

thermal_frame

And the phone records:

RGB frame

LiDAR frame

GPS

timestamp

Critical: synchronize everything using timestamps.

You don't need sophisticated sensor fusion initially.

# Program #2 — Crack/pothole detector

This is where I'd use a YOLO-style computer vision model.

Don't ask an LLM to inspect every road image.

Instead:

Image

↓

YOLO

↓

bounding boxes / segmentation

↓

crack

pothole

patch

raveling

For the competition, start with:

3 classes

CRACK

POTHOLE

OTHER_DISTRESS

You can expand later.

Recent pavement research continues to use YOLO-family models for pavement distress and deformation detection.

# Program #3 — LiDAR deformation detector

This is potentially your coolest demo.

Take your point cloud:

x

y

z

Create a reference road surface.

Then calculate:

ΔZ = observed elevation - expected elevation

For example:

ROAD

0mm  0mm  -2mm  -4mm

│    │     │     │

───┴────┴─────┴─────┴──

↓

-38 mm

↓

───\____/───

RUT

Output:

Rut detected: 38 mm

That is much more compelling than simply drawing a box around a pothole.

# Program #4 — Thermal anomaly detector

You don't even need AI initially.

Take the thermal image.

Calculate:

local_temperature

-

surrounding_temperature

Then:

if temperature_difference > threshold:

anomaly = True

Create:

NORMAL      NORMAL       ANOMALY

████████    ████████     ████🔴████

████████    ████████     ███🔴🔴███

████████    ████████     ████🔴████

Then overlay it onto the RGB image.

# Program #5 — Sensor fusion

This is where your project becomes much more interesting.

Don't simply say:

AI found a pothole.

Instead:

VISION

Pothole probability = 91%

+

LiDAR

Depth = 42 mm

+

THERMAL

Temperature anomaly = +3.2°C

+

IMU

Vehicle vibration = 2.1σ above baseline

↓

MULTIMODAL EVENT

↓

HIGH-PRIORITY INSPECTION

Now you have a multimodal infrastructure inspection system.

# Program #6 — GIS dashboard

I'd build this with Streamlit + Python.

Screen:

┌──────────────────────────────────────────────┐

│          ROAD CONDITION MONITOR              │

├──────────────────────────────────────────────┤

│                                              │

│            MAP                               │

│                                              │

│       🟢🟢🟡🔴🟢🟢                          │

│           🟡     🔴                          │

│                                              │

├──────────────────────────────────────────────┤

│ DETECTED DISTRESS                            │

│                                              │

│ 🔴 Pothole       42 mm       HIGH            │

│ 🟡 Crack          1.8 m       MEDIUM          │

│ 🟡 Rut            31 mm       MEDIUM          │

│ 🔴 Thermal        +3.2°C     HIGH            │

│                                              │

└──────────────────────────────────────────────┘

Clicking the red point opens:

LOCATION #037

Pothole

Depth: 42 mm

Area: 0.31 m²

Thermal anomaly: YES

LiDAR anomaly: YES

IMU anomaly: YES

Confidence: 94%

Recommended:

Targeted pavement engineering inspection

# What LLMs should you use?

This is important:

Don't use an LLM as the primary defect detector.

Use specialized computer vision models for that.

Use LLMs as your engineering/coding agents.

### My stack

| Job | Tool |
| --- | --- |
| Coding | GPT-5.6 Luna / ChatGPT |
| Debugging | GPT-5.6 Luna |
| Architecture | GPT-5.6 Luna |
| Computer vision | YOLO + OpenCV |
| Image segmentation | YOLO segmentation / SAM-style model |
| Point cloud | Open3D |
| Data analysis | Python + NumPy/Pandas |
| Dashboard | Streamlit |
| Maps | Mapbox/Leaflet/Plotly |
| Database | SQLite |
| Thermal processing | NumPy/OpenCV |
| Sensor interface | Python GPIO/I²C |
| Documentation | LLM |

If you have access to multiple models, you can use one LLM for code generation and another as a reviewer/debugger, but don't spend your three days building a complicated multi-agent architecture.

# Hardware budget

You can actually make this relatively cheap.

### If you already have an iPhone Pro

| Component | Approx. |
| --- | --- |
| Raspberry Pi Zero 2 W | ~$15–30 |
| Camera | ~$30–40 |
| MLX90640 | ~$70–100 |
| MPU6050 | ~$5–15 |
| GPS | ~$15–30 |
| MicroSD | ~$10 |
| Battery/power bank | ~$15–25 |
| Wires/breadboard | ~$10 |
| Mount | ~$10–20 |
| Total | ~$180–285 |

A current Pi Zero 2 W listing I found is around $14–15 from one specialist retailer, while other marketplaces can be dramatically more expensive, so don't pay $90+ for one. Raspberry Pi Zero 2 W

You can also skip the separate Pi camera entirely if the iPhone handles RGB.

That gets you closer to:

### ~$100–180 incremental hardware

because your iPhone provides the primary camera/LiDAR/GPS.

# But here's the version I'd actually build

## Hardware

IPHONE PRO

┌────────────────┐

│ RGB CAMERA     │

│ LiDAR          │

│ GPS            │

│ IMU            │

└───────┬────────┘

│ WiFi/Bluetooth

↓

RASPBERRY PI ZERO 2 W

┌──────────────────────┐

│ MLX90640 Thermal     │

│ MPU6050 IMU           │

│ Data Logger            │

└──────────┬───────────┘

│

↓

LAPTOP

│

┌──────────┴──────────┐

│                     │

AI ENGINE            GIS ENGINE

│                     │

YOLO/OpenCV         Open3D/Map

│                     │

└──────────┬──────────┘

↓

DASHBOARD

# Your 3-day schedule

## DAY 1 — Make sensing work

### Morning

Get:

- Pi running

- camera working

- thermal sensor working

- IMU working

- iPhone LiDAR scan working

Don't touch AI yet.

### Afternoon

Write one Python collector:

collect.py

which outputs:

/data/

timestamp.csv

thermal/

images/

imu/

lidar/

### Evening

Build the basic dashboard.

# DAY 2 — AI + geometry

Build:

### Vision

image → YOLO → distress

### LiDAR

point cloud → road plane → ΔZ → rut/pothole depth

### Thermal

thermal → temperature map → anomaly

Get all three working independently.

# DAY 3 — The "wow" factor

Don't add more sensors.

Fuse what you already have.

Demo:

### Scan #1

Healthy road.

🟢 NORMAL

### Modify your test road

Put:

- fake pothole

- crack

- depressed section

- warm/cold material

- patch

Then scan again.

Your system says:

ROAD SEGMENT 014

VISION

🔴 Pothole detected

Confidence: 93%

LiDAR

🔴 Depression: 41 mm

THERMAL

🟡 Temperature anomaly: +2.8°C

IMU

🟡 Vibration anomaly

────────────────────────

MULTIMODAL ANOMALY SCORE

87 / 100

PRIORITIZE INSPECTION

That is your demo.

# The really important product insight

I'd actually change your wording from:

"Structural inspection of asphalt."

to:

"Automated pavement condition and deterioration monitoring."

Because your system can legitimately demonstrate:

Surface condition

→ cracks / potholes

Geometric condition

→ rutting / settlement / deformation

Thermal condition

→ anomalies / potential distress

Dynamic condition

→ vibration / roughness

Historical condition

→ how all of those measurements change over time.

The last one is particularly powerful.

# Your eventual product

Imagine the same road scanned:

January

Crack = 1.2 m

Rut = 8 mm

April

Crack = 2.1 m

Rut = 13 mm

July

Crack = 4.8 m

Rut = 27 mm

Your system recognizes:

Deterioration accelerating at Segment 143.

Then the city doesn't have to inspect every mile equally.

It can prioritize the specific road segments showing measurable deterioration.

That is a much stronger long-term concept than simply "Raspberry Pi detects potholes."

### One more important limitation

For your competition, call the output "inspection priority" or "condition anomaly," not "structural failure." Surface imagery and LiDAR are excellent for screening, but they cannot by themselves establish subsurface pavement structural capacity. Current reviews explicitly distinguish 2D visual, 3D geometric, and subsurface sensing such as GPR for this reason.

If you have exactly 3 days, I'd put roughly 70% of your effort into RGB + LiDAR + dashboard, 20% into thermal/IMU, and 10% into the Raspberry Pi sensor integration. That gives you the highest probability of having a working end-to-end demonstration rather than five half-working sensors.

how can i make this at low input and automated as possible?

Yes. If your goal is low human input + highly automated inspection, I would change the concept from a “sensor that an inspector operates” into a drive-over autonomous pavement monitoring system.

The basic idea:

Mount one sensor box on a car → drive normally → automatically capture road data → AI detects anomalies → GPS maps them → system prioritizes what needs inspection.

## The lowest-input architecture

🚗 NORMAL VEHICLE

│

┌────────▼────────┐

│ SENSOR BOX      │

│                 │

│ Camera          │

│ LiDAR           │

│ IMU             │

│ Thermal         │

│ GPS             │

└────────┬────────┘

│

ONE BUTTON

│

▼

AUTOMATIC CAPTURE

│

▼

AI PROCESSING

│

┌────────────┼────────────┐

▼            ▼            ▼

SURFACE       GEOMETRY     MOTION

ISSUES        ISSUES       ISSUES

│            │            │

└────────────┼────────────┘

▼

SENSOR FUSION

│

▼

GPS ROAD MAP

│

▼

PRIORITY QUEUE

│

▼

🚨 INSPECT THESE

LOCATIONS

The human should ideally only do:

Mount → Start → Drive.

Everything else happens automatically.

# 1. Don't make the Raspberry Pi the brain

This is probably the biggest architectural change I'd make.

Use:

### Raspberry Pi

Sensor controller

### Phone

Camera + LiDAR + GPS + user interface

### Cloud/laptop

AI + heavy processing

That keeps the physical device cheap and simple.

# 2. Make the sensor box completely self-contained

Build something like:

┌──────────────────────────┐

│      ROADSCAN BOX        │

│                          │

│     📷 Camera            │

│                          │

│     🌡️ Thermal           │

│                          │

│     📡 LiDAR             │

│                          │

│     📳 IMU               │

│                          │

│     GPS                  │

│                          │

│     Raspberry Pi         │

│                          │

│     Battery              │

└────────────┬─────────────┘

│

Wi-Fi/5G

│

▼

Cloud

Mount it to the roof/front of the vehicle.

You don't want someone holding sensors.

# 3. Automatic triggering

You don't want:

"Press button every time you see a pothole."

Instead:

### Continuous capture

Camera:

30 FPS

LiDAR:

continuous scanning

IMU:

100+ Hz

GPS:

1–10 Hz

Thermal:

several frames/sec

Then software automatically decides which data is worth saving.

# 4. Use the IMU to trigger higher-resolution inspection

This is a clever way to reduce computing requirements.

Suppose the car normally produces:

Acceleration

───────────────

normal

Then:

──────────╱╲────────

↑

big impact

The Pi recognizes:

Large vertical acceleration detected.

Then automatically tells the camera:

Save high-resolution frames from ±3 seconds around this event.

So you're not processing every frame equally.

# 5. Automatically detect road anomalies

You can create several independent detectors.

### Vision

Camera

↓

YOLO

↓

crack

pothole

patch

raveling

### LiDAR

Point cloud

↓

road surface model

↓

elevation deviation

↓

rut / pothole / settlement

### IMU

Acceleration

↓

filter noise

↓

detect spikes

↓

roughness event

### Thermal

Thermal image

↓

compare neighboring pavement

↓

temperature anomaly

# 6. Then automatically combine them

This is where I'd make your system much more sophisticated.

Don't have an LLM decide whether something is dangerous.

Create a deterministic condition engine.

For example:

VISION

Pothole: 92%

+

LIDAR

Depth: 43 mm

+

IMU

Impact: 3.1σ

+

THERMAL

Anomaly: YES

↓

CONDITION ENGINE

↓

Priority = HIGH

The AI/LLM can then explain the result:

"A 43 mm depression was detected at this location. Computer vision independently identified a pothole and vehicle acceleration exceeded the local baseline."

# 7. Automatically create a road "health score"

Break the road into segments.

For example:

every 10 meters.

Road

─────────────────────────────────────────

0      10      20      30      40      50m

🟢      🟢      🟡      🔴      🟢      🟡

Each segment gets:

Segment 30

Cracking       72/100

Potholes       91/100

Rutting        84/100

Roughness      76/100

Thermal        44/100

Overall condition:

HIGH PRIORITY

Now your system can automatically produce a maintenance list.

# 8. Make the system learn the road's baseline

This is probably the most valuable automation feature.

Don't rely entirely on generic thresholds.

The first time the car drives a road:

### Baseline

Segment 100

Elevation = baseline

Acceleration = baseline

Temperature = baseline

Cracking = baseline

Next drive:

Segment 100

Elevation change = -11 mm

Acceleration = +18%

Crack length = +0.8m

The system automatically says:

Deterioration detected.

You don't need an engineer manually comparing two scans.

# 9. Repeated autonomous monitoring

This is where it becomes a real product.

You could put your device on:

- garbage trucks

- street sweepers

- city maintenance vehicles

- buses

- delivery fleets

- utility vehicles

These vehicles are already driving roads every day.

That means you don't need to send an inspection vehicle everywhere.

Imagine:

CITY FLEET

🚛 Garbage Truck #12

↓

47 miles

🚌 Bus #34

↓

82 miles

🚚 Utility Truck #8

↓

31 miles

All of them continuously collect road-condition data.

Your software merges everything.

CITY ROAD DATABASE

8,421 miles

│

▼

14,231 segments

│

▼

438 anomalies

│

▼

37 high-priority

│

▼

MAINTENANCE QUEUE

That is much more scalable than dedicated inspectors.

# 10. The automation loop

I'd make your entire product revolve around this:

### IDENTIFY

AI automatically detects:

- crack

- pothole

- rut

- deformation

- unusual vibration

- thermal anomaly

↓

### LOCATE

GPS automatically assigns:

Lat

Lon

Road

Lane

Segment

↓

### VERIFY

Multiple sensors cross-check the anomaly.

↓

### PRIORITIZE

System calculates:

severity

+

confidence

+

rate of deterioration

+

traffic exposure

↓

### ACTION

Automatically generates:

Maintenance ticket #1842

Location:

Segment 38

Issue:

Pothole

Depth:

43 mm

Detected:

Sept 25

Priority:

High

Evidence:

Camera ✓

LiDAR ✓

IMU ✓

Suggested action:

Targeted pavement inspection

# 11. You can even automate the maintenance side

This is where your previous infrastructure idea can become much bigger.

Don't stop at:

"We found a pothole."

Build:

Detect → Predict → Prioritize → Dispatch → Verify

ROAD DATA

↓

DETECT

↓

PREDICT

↓

PRIORITIZE

↓

DISPATCH

↓

MAINTENANCE CREW

↓

REPAIR

↓

DRIVE OVER AGAIN

↓

VERIFY

After the crew repairs it:

Before:

🔴 Pothole 43 mm

After:

🟢 Surface restored

Your vehicle becomes the automated verification system.

# For your 3-day competition

I would simplify this dramatically.

## Build only this:

### Hardware

iPhone Pro + Raspberry Pi + MLX90640 + IMU

No custom ultrasonic.

No GPR.

No drone.

No robotics.

### Software

Build 5 programs:

1. collector.py

↓

2. vision_detector.py

↓

3. lidar_analyzer.py

↓

4. sensor_fusion.py

↓

5. dashboard.py

### One drive

Drive over a controlled test road.

Your system automatically generates:

ROADSCAN REPORT

Distance scanned: 0.8 miles

Segments analyzed: 127

Cracks: 18

Potholes: 7

Rutting events: 4

Thermal anomalies: 3

Vibration anomalies: 9

MULTI-SENSOR CONFIRMED:

5

HIGH PRIORITY:

2

──────────────

LOCATION #47

Pothole

Depth: 41 mm

Camera: 94%

LiDAR: confirmed

IMU: confirmed

→ Maintenance inspection recommended

That would be a very compelling 3-day MVP because the judge sees the entire pipeline rather than a cool sensor sitting on a table.

### And your long-term pitch becomes:

“We don't send inspectors to find problems. We turn vehicles already driving the roads into autonomous infrastructure inspectors.”

That's the direction I'd take if your goal is maximum automation with minimum human input.

Absolutely. If you structure the opportunity around the four verbs in the prompt — identify → predict → automate inspection → prioritize — there are a lot of directions beyond “car-mounted crack detector.”

The most interesting ideas use multiple existing data streams so you're solving the problem of too much data + too much manual work, rather than just adding another sensor.

## 1. AI “Inspector Agent”

Instead of just detecting damage, an AI agent manages the entire inspection workflow.

Data comes in:

- Drone imagery

- Vehicle cameras

- Satellite imagery

- IoT sensors

- Previous inspection reports

- Weather

- Seismic events

- GIS

Then:

AI Agent
 → identifies anomaly
 → checks historical records
 → determines whether anomaly is new
 → estimates severity
 → determines inspection type needed
 → creates work order
 → assigns inspection crew
 → tracks completion
 → updates asset history

The human becomes the exception handler, rather than manually managing every inspection.

# 2. Autonomous “inspection scheduling”

This is different from damage detection.

The AI constantly asks:

What should we inspect tomorrow?

It considers:

- Asset age

- Condition

- Previous inspection

- Recent deterioration

- Weather

- Earthquakes

- Traffic/load

- Failure consequences

- Inspector availability

- Geographic proximity

Then automatically creates:

Tomorrow's inspection route

For example:

Crew 1

8:00 AM → Bridge A

9:15 AM → Bridge C

10:00 AM → Road D

11:30 AM → Utility Pole F

Reason:

High deterioration + recent seismic activity

This directly attacks the manual planning problem.

# 3. AI route optimization for inspectors

Take it one step further.

Instead of:

“Which assets need inspection?”

ask:

“What's the most efficient way to inspect everything that needs inspection?”

The system combines:

Risk × location × inspection type × crew skills × traffic × travel time

and generates optimized routes.

This could save substantial field time without requiring better sensors.

# 4. “Inspection by exception”

This is probably one of the strongest concepts.

Instead of inspecting every asset equally:

### Normal

AI sees:

No meaningful change.

→ No inspection required

### Uncertain

Something changed.

→ Quick visual inspection

### High risk

Multiple independent signals indicate deterioration.

→ Engineer/NDT inspection

So you're turning:

100% manual inspection

into:

AI screening → human inspection only where necessary.

# 5. Multi-agent infrastructure monitoring

You could have specialized AI agents:

### 👁️ Vision Agent

Finds visible damage.

### 📡 Sensor Agent

Analyzes vibration/temperature/acoustic data.

### 🌎 Geospatial Agent

Analyzes location, terrain, flooding, seismic exposure.

### 📚 History Agent

Reads inspection reports and maintenance records.

### 🔮 Prediction Agent

Estimates deterioration.

### 🚨 Response Agent

Prioritizes and creates work orders.

Then they collaborate:

Vision Agent:

"Crack detected."

Sensor Agent:

"Vibration increased 31%."

Geospatial Agent:

"Earthquake occurred 2.8 miles away."

History Agent:

"Same location had corrosion 14 months ago."

Prediction Agent:

"Deterioration probability increased."

Response Agent:

"Schedule inspection within 72 hours."

That is much more compelling than one generic AI model.

# 6. Automatically determine what kind of inspection is needed

This is an overlooked problem.

Not every anomaly requires the same response.

AI could classify:

| Detection | Recommended inspection |
| --- | --- |
| Surface crack | Visual/CV |
| Suspected delamination | Thermal/ultrasound |
| Ground movement | LiDAR/GPS |
| Excessive vibration | Accelerometer |
| Corrosion | NDT |
| Possible internal concrete defect | GPR/ultrasound |
| Bearing anomaly | Specialized structural inspection |

So instead of simply:

“Something is wrong.”

the system says:

“Something is potentially wrong, and this is the cheapest appropriate way to investigate it.”

# 7. Autonomous “second opinion”

Imagine an engineer receives an inspection report.

AI independently analyzes:

- Images

- Measurements

- Previous reports

- Sensor data

and says:

“Three defects in this inspection may warrant additional review.”

This is useful because human inspections themselves can be inconsistent.

You're not replacing the engineer; you're providing quality control.

# 8. Automatically compare every inspection to every previous inspection

This could be extremely valuable.

Today:

Inspector looks at 2026 photos

and may need to manually compare them with:

2025 photos + 2024 report + 2023 report.

AI does it automatically.

2024

██████

2025

████████

2026

████████████

Then:

Crack length increased ~40% over 24 months.

The key is longitudinal infrastructure intelligence.

# 9. Automatically detect “silent deterioration”

Some infrastructure problems aren't obvious visually.

You could look for behavioral changes.

For example:

Normal vibration pattern

████████████

Current vibration

██████████████████

The system flags:

Behavior deviates from historical baseline.

Potentially useful for:

- Bridges

- Railways

- Machinery

- Pipelines

- Industrial structures

- Utility infrastructure

This gets you beyond pure computer vision.

# 10. Crowdsource infrastructure inspection

Turn every smartphone into an inspection device.

Someone records a video while walking/driving.

AI extracts:

- Location

- Images

- Defects

- Road condition

- Infrastructure type

Then automatically merges observations.

If 50 people independently report the same location, confidence increases.

Citizen 1 ─┐

Citizen 2 ─┤

Bus camera ├──→ AI → confirmed anomaly

Drone ─────┤

Satellite ─┘

This creates a distributed infrastructure sensor network.

# 11. Vehicles as “inspection swarms”

Instead of one inspection vehicle:

100 existing vehicles collect data.

For example:

Bus 17 ─────┐

Bus 23 ─────┤

Delivery 4 ─┤

Taxi 82 ────┤──→ Infrastructure AI

Garbage 11 ──┤

Police 7 ────┘

You don't need dedicated inspection trips.

The system piggybacks on existing mobility.

That's a particularly strong answer to the prompt's emphasis on the unprecedented amount of data.

# 12. Predict where failures will happen

Instead of predicting:

“Bridge A will fail.”

(which is extremely difficult)

predict:

“These areas have an unusually high deterioration trajectory.”

Use:

Asset characteristics + environmental conditions + historical deterioration + sensor signals + traffic + weather.

Then create a heatmap:

LOW RISK

🟢🟢🟢🟢

MEDIUM

🟡🟡🟡

HIGH

🔴🔴

This is much more realistic.

# 13. AI maintenance budget allocator

Now you're attacking the prioritize component directly.

Suppose an agency has:

$10M maintenance budget

and:

2,000 assets.

AI produces scenarios:

### Scenario A

Fix 8 major assets.

### Scenario B

Fix 30 moderate assets.

### Scenario C

Preventative maintenance across 100 assets.

Then show:

Expected risk reduction / dollar spent.

This turns infrastructure inspection into a capital allocation problem.

# 14. “Digital twin that updates itself”

Create a digital representation of the infrastructure.

But instead of a static digital twin:

Every new observation updates it automatically.

BRIDGE 182

Structure

├── Deck

├── Bearings

├── Girders

├── Piers

└── Foundation

Data

├── 2024 inspection

├── 2025 drone scan

├── 2026 vehicle scan

├── Sensor data

└── Seismic events

AI continuously updates the condition of each component.

# 15. Disaster mode

This is where your everyday system becomes extremely powerful.

Normal day:

Continuously monitor.

Earthquake happens:

SYSTEM SWITCHES TO DISASTER MODE

Immediately:

- Identify affected assets

- Compare before/after imagery

- Analyze sensor anomalies

- Estimate damage

- Rank assets

- Generate inspection routes

- Dispatch crews

- Continuously update the map

So instead of having a separate disaster product:

Your normal infrastructure monitoring system becomes an emergency response system.

# 16. The really ambitious version: “self-driving inspection”

Eventually:

Autonomous vehicle

↓

Infrastructure scanning

↓

AI detects anomaly

↓

Vehicle automatically slows down

↓

Captures additional imagery

↓

Changes sensor configuration

↓

Uploads evidence

↓

AI determines inspection priority

The vehicle itself decides:

“I saw something unusual. I'm going to collect more data here.”

That's an exciting long-term vision, even if your competition prototype only demonstrates the first pieces.

# How I'd organize your competition concept

You could frame the whole platform as:

## IDENTIFY → PREDICT → INSPECT → PRIORITIZE

### 👁️ IDENTIFY

What is wrong?

Camera + thermal + LiDAR + vibration + satellite

↓

### 🔮 PREDICT

What is getting worse?

Historical observations + environmental data + sensor trends

↓

### 🤖 AUTOMATE INSPECTION

What needs a human and what doesn't?

AI screening + inspection scheduling + route optimization + work orders

↓

### 🎯 PRIORITIZE

Where should limited resources go?

Risk × consequence × uncertainty × cost

## And your Raspberry Pi fits beautifully

Don't make the Raspberry Pi the product.

Make it your physical-world data collection node.

PHYSICAL WORLD

↓

┌────────────────────────┐

│ MOBILE SENSOR NODE     │

│                        │

│ Camera                 │

│ IMU                    │

│ GPS                    │

│ Thermal                │

│ Optional ultrasonic   │

└───────────┬────────────┘

↓

RAW DATA

↓

┌────────────────────────┐

│ AI INFRASTRUCTURE      │

│                        │

│ Identify               │

│ Predict                │

│ Compare                │

│ Prioritize             │

└───────────┬────────────┘

↓

INSPECTION QUEUE

↓

┌────────────────────────┐

│ "GO LOOK HERE FIRST"   │

└────────────────────────┘

For the 3-day competition, I'd build the smallest convincing version of that loop: a Pi-mounted camera + IMU + GPS, automated data collection, CV anomaly detection, GPS mapping, and an AI-generated ranked inspection queue. Then present autonomous inspection scheduling, multi-source sensor fusion, and predictive maintenance as the expansion path.

Go.AI, On-Premises AI for Regulated Industries

Private AI On-Prem

Deploy secure AI in your own environment.

Ad

More options

how would this be best for roads (asphalt?) over just dirving over it?

Yes. Roads/asphalt may actually be the easiest place to make the Raspberry Pi vehicle concept work in 3 days, because you can detect several meaningful surface-level problems simply by driving over/along the road.

The key is to distinguish “drive over it and measure it” from “drive over it and somehow see through the asphalt.”

## Best road setup

I'd build a mobile road-condition scanner:

CAR

┌──────────────────┐

│ Raspberry Pi     │

│                  │

│ 📷 Camera ───────┼──→ road ahead

│                  │

│ 📡 IMU ──────────┤

│                  │

│ 📍 GPS ──────────┤

│                  │

│ 🌡 Thermal ──────┤

└──────────────────┘

↓

ASPHALT

### The most important sensor isn't ultrasound.

For asphalt, I'd prioritize:

Camera > IMU > GPS > LiDAR/ToF > thermal > ultrasound

# 1. Camera: detect what's actually visible

Point the camera toward the road.

You can identify:

- Potholes

- Cracks

- Alligator cracking

- Rutting

- Patches

- Faded markings

- Surface deterioration

- Debris

- Standing water

- Manhole abnormalities

For example:

CAMERA

↓

┌─────────────────────┐

│       asphalt       │

│                     │

│    ╲ ╱              │

│     ╲     crack     │

│      ╲              │

│          ○ pothole  │

└─────────────────────┘

The important thing is that the AI doesn't just say “pothole.”

It records:

Pothole → location → size → severity → image → time

# 2. IMU: detect what the car physically experiences

This is where your concept gets much more interesting.

Put an accelerometer/IMU on the vehicle.

When you drive over:

### Normal asphalt

acceleration

│

│   ~~~~~~~~

│ ~~~~~~~~~~~~

└──────────────→ time

### Pothole

acceleration

│

│      /\

│     /  \

│____/    \________

└──────────────────→ time

You can detect:

vertical acceleration spike

and associate it with the GPS location.

So you have two independent signals:

Camera: “I see a pothole.”

IMU: “The vehicle experienced a sharp impact.”

Together → higher confidence.

# 3. You can actually use the vehicle's suspension as a sensor

This is a really cool angle for the pitch.

You don't necessarily need sophisticated infrastructure sensors.

The car is already interacting with the road.

The road causes:

vertical acceleration + vibration + pitch + roll

Your system can measure those.

You can essentially turn the vehicle into a mobile road profiler.

# 4. GPS turns measurements into infrastructure intelligence

Every anomaly gets a coordinate.

For example:

08:32:11

34.05281, -118.2432

Acceleration: +3.4g

Camera: pothole 91%

Then:

08:37:42

34.05401, -118.2418

Acceleration: +2.7g

Camera: crack 84%

Put these on a map.

Now you've created:

## Road condition heatmap

7

🟢 🟢 🟢

🟢       🟡

🔴

🟡

🟢              🔴

# 5. The really cool part: repeated drives

Suppose a city bus drives the same road every day.

### Day 1

Road segment 184

Condition: 82

### Day 30

Condition: 75

### Day 60

Condition: 61

AI says:

Rapid deterioration detected.

You don't necessarily need to understand the internal mechanics of the asphalt.

You're identifying a changing physical signature.

# 6. You can detect rutting

Rutting is when the wheel paths become depressed.

This is where a LiDAR/ToF sensor becomes useful.

You could mount one looking down at the road.

Conceptually:

Normal road:

_________

/         \

_____/           \_____

Rutted road:

___________

______/           \____

\___________/

A distance sensor can detect the road profile.

With multiple measurements across the road, you can estimate surface deformation.

For a 3-day prototype, though, I wouldn't make this your primary feature unless you already have the hardware.

# 7. Thermal is more of a secondary signal

A thermal camera can potentially identify:

- Temperature differences

- Water/moisture-related anomalies

- Different surface materials

- Some pavement deterioration patterns

But don't pitch:

“Thermal camera detects structural failure.”

That's too strong.

Instead:

Thermal data provides another signal that can help identify unusual pavement conditions.

# 8. Ultrasound is the wrong first move for asphalt

This is important.

A cheap ultrasonic distance sensor:

HC-SR04

basically asks:

“How far away is the surface?”

It doesn't ask:

“What's happening 10 cm beneath the asphalt?”

Professional ultrasonic testing uses very different transducers, frequencies, coupling methods, signal processing, etc.

So for your competition:

### Don't spend your 3 days trying to make cheap ultrasound inspect asphalt internally.

Use:

Camera + IMU + GPS.

You can add thermal or distance sensing if you have time.

# 9. Make the inspection completely automatic

The vehicle doesn't need an inspector sitting there.

You could have:

CAR STARTS

↓

Pi starts recording

↓

GPS continuously records location

↓

Camera continuously records road

↓

IMU continuously records vibration

↓

AI analyzes data

↓

Anomaly detected?

↓

YES

↓

Capture high-resolution image

↓

Save GPS coordinate

↓

Calculate severity

↓

Add to inspection queue

At the end:

“4.2 miles scanned.”

“17 anomalies detected.”

“3 require immediate inspection.”

# 10. You can make it much smarter than just detecting potholes

Imagine:

### Camera

Crack probability: 92%

### IMU

Vibration anomaly: 2.8σ

### Historical data

Same location flagged 3 times

### Weather

Heavy rainfall last 48 hours

AI:

🔴 HIGH PRIORITY

That is much more powerful than:

“I found a pothole.”

# 11. Your actual product could be “Road Health Score”

Divide the road into segments:

100m segment

and calculate:

Road Health =

surface defects

+ vehicle vibration

+ deterioration trend

+ environmental exposure

+ historical observations

Then:

| Road Segment | Health | Action |
| --- | --- | --- |
| A | 91 | Monitor |
| B | 76 | Routine maintenance |
| C | 43 | Inspect |
| D | 21 | Immediate inspection |

Again, call it a screening/priority score, not an engineering safety certification.

# 12. The best 3-day prototype

I would not attempt to build a sophisticated sensor array.

Build this:

### Hardware

Raspberry Pi

- Pi Camera

- MPU6050/other IMU

- GPS module

- Optional thermal camera

### Software

Camera

↓

Computer vision

↓

Crack / pothole detection

IMU

↓

Vibration analysis

↓

Road roughness / impact detection

GPS

↓

Geospatial coordinates

↓

SENSOR FUSION

↓

ROAD CONDITION MAP

↓

PRIORITIZED INSPECTION LIST

### Demo

Drive over:

- Normal asphalt

- Crack

- Pothole/bump

- Rough pavement

And show your dashboard automatically change:

SCANNING...

████████████████ 100%

4.1 miles scanned

1,823 observations

23 anomalies

🔴 3 HIGH PRIORITY

🟡 7 MEDIUM

🟢 13 LOW

Then click one:

Road Segment 43

📍 GPS location
 📷 Pothole detected — 93% confidence
 📈 Vehicle impact — 3.2σ above baseline
 📅 Previously flagged 21 days ago
 📉 Condition deteriorating

Recommendation: Dispatch maintenance crew

That is a very compelling competition prototype because you're demonstrating the entire loop:

drive → automatically collect → identify → corroborate → geolocate → detect deterioration → prioritize → dispatch.

ideas to provide automated inspections and how to take action with automated maintencance and recovery resources within the 3 days competition

Yes. For a 3-day competition, I would shift the prototype from “AI detects road damage” to “AI autonomously turns detection into a maintenance action.”

The impressive part of the demo should be:

Something gets detected → the system decides what inspection is needed → creates the work order → assigns the right resource → optimizes the route → closes the loop.

You don't need actual autonomous road repair. You need to automate the decision and dispatch workflow.

## 1. Automated inspection-on-demand

Instead of a fixed inspection schedule:

Vehicle detects anomaly

→ AI checks confidence
 → checks whether it has been seen before
 → determines severity
 → automatically creates inspection request.

Example:

🔴 Road Segment #143

Pothole detected: 91%
 Vehicle impact: 3.4σ
 Previously detected: No
 Traffic volume: High

Action: Create inspection work order.

Then the system generates:

Inspection #4821

- Location: GPS coordinate

- Issue: Suspected pavement failure

- Required equipment: Camera + measurement

- Priority: High

- Deadline: 24 hours

This is very achievable in 3 days.

# 2. AI decides whether a human inspection is even necessary

This is an important automation layer.

DETECTED

↓

AI CONFIDENCE

↓

┌───────────┴───────────┐

↓                       ↓

HIGH CONFIDENCE         UNCERTAIN

↓                       ↓

Automated action       Human inspection

↓

Maintenance

For example:

### Small crack

Monitor automatically.

### Large pothole

Create maintenance request.

### Major anomaly + abnormal vibration

Send inspector.

### Multiple signals + critical road

Escalate immediately.

You're automating triage.

# 3. Automated inspection route generation

This could be one of the easiest impressive features.

Suppose your system finds:

17 anomalies

Instead of giving an engineer a list, it creates:

Today's inspection route

DEPOT

↓

🔴 #17

↓

🔴 #4

↓

🟡 #11

↓

🔴 #2

↓

🟡 #9

↓

DEPOT

Optimize based on:

- Severity

- Distance

- Traffic

- Crew availability

- Required equipment

- Estimated inspection duration

You can implement this with a basic routing algorithm/API.

# 4. Automated crew assignment

Now add a resource layer.

Imagine you have:

| Crew | Capability | Location |
| --- | --- | --- |
| Crew A | Pavement | 2 mi |
| Crew B | Structural | 8 mi |
| Crew C | Electrical | 4 mi |

AI determines:

Crew A → Road #17

rather than simply notifying everyone.

For bridges:

Structural Crew B → Bridge #4

For a power line:

Electrical Crew C → Pole #82

This directly addresses “rapidly prioritize recovery.”

# 5. Automated maintenance work orders

Once an anomaly passes a threshold:

AI

↓

Maintenance required

↓

Generate work order

Your prototype dashboard could show:

### WORK ORDER #1847

Asset: Road Segment 43
 Problem: Pothole
 Severity: High
 Coordinates: 34.x, -118.x
 Evidence: Image + IMU
 Recommended action: Patch pavement
 Estimated duration: 45 min
 Assigned crew: Crew A
 ETA: 18 min

Then have a button:

DISPATCH

You don't actually need to connect to a city's maintenance system. Your prototype can simulate the downstream action.

# 6. Automated maintenance recommendation

Don't just detect the problem.

Recommend what to do.

For example:

| Detection | AI Recommendation |
| --- | --- |
| Small crack | Monitor |
| Large crack | Seal |
| Pothole | Patch |
| Extensive cracking | Resurface |
| Severe deformation | Engineer inspection |
| Possible subsurface issue | NDT inspection |
| Flood damage | Drainage assessment |

The system can use a simple rules engine for the competition.

You can honestly describe this as:

AI-assisted maintenance decisioning

rather than pretending your model has engineering authority.

# 7. Automated recovery prioritization

This is where the disaster component comes in.

Imagine an earthquake occurs.

Your system receives:

Earthquake location

↓

Automatically identifies infrastructure within the affected radius.

EARTHQUAKE

↓

Affected assets

↓

AI damage screening

↓

Risk ranking

Then:

### Recovery Queue

1 🔴 Bridge A

Critical route

Suspected damage: 86%

2 🔴 Road B

Hospital access

Damage probability: 74%

3 🟡 Road C

Moderate traffic

Damage probability: 51%

4 🟢 Road D

Low traffic

Damage probability: 12%

Now you aren't just detecting damage.

You're answering:

“Where should the first recovery crew go?”

# 8. Add criticality to your prioritization

This makes the system much more realistic.

A pothole on a quiet residential street isn't equivalent to one on an emergency route.

Your score could be:

Priority =

Damage likelihood × severity × infrastructure criticality × traffic × uncertainty

For example:

Pothole A

Damage: 80

Traffic: 20

Criticality: 20

→ Medium priority

Pothole B

Damage: 70

Traffic: 95

Criticality: 90

→ HIGH priority

That is a much better story than simply ranking by damage severity.

# 9. Automated “what if?” resource allocation

This could be a killer dashboard feature.

Suppose the city has:

5 crews

and:

37 damaged locations.

Your system determines:

If we dispatch these five crews first, we address 68% of estimated infrastructure risk.

Then the user can change:

Available crews: 5 → 2

and the system recalculates.

Or:

Budget: $100k → $50k

and it generates a different maintenance plan.

You're effectively creating an infrastructure recovery optimizer.

# 10. Automatic escalation

You can create different levels:

### 🟢 Monitor

No action.

### 🟡 Maintenance queue

Routine repair.

### 🟠 Inspection

Human/technical inspection required.

### 🔴 Emergency

Immediate response.

Then your system automatically moves assets between levels.

Example:

Monday

🟢 Crack

↓

Wednesday

🟡 Crack worsening

↓

Friday

🟠 Significant deterioration

↓

After earthquake

🔴 Emergency inspection

This demonstrates continuous infrastructure monitoring.

# 11. Close the loop automatically

This is something I'd definitely include.

Most systems stop at:

“We found a problem.”

Your system should go:

Detect

↓

Inspect

↓

Repair

↓

Verify

↓

Learn

For example:

### Before

AI detects pothole.

### Action

Crew patches it.

### Verification

Next vehicle drives over the same location.

AI sees:

Road anomaly disappeared.

### System

Work order automatically verified as complete.

That is an extremely strong automation story.

# 12. “Proof of repair”

Your inspection vehicle can automatically verify maintenance.

Before:

🔴 Pothole

After repair:

🟢 Normal surface

The system can compare:

Before image vs. after image

and:

Before IMU vs. after IMU

Then:

Repair successful — automatically verified.

This reduces another manual process.

# 13. Have the vehicle automatically trigger a second inspection

This is another clever feature you can build quickly.

Suppose the first pass isn't conclusive:

CV confidence = 61%

Instead of asking an employee to decide what to do:

61% confidence

↓

AI requests additional scan

↓

Vehicle slows down

↓

Captures 10 additional images

↓

Reanalyzes

Now your inspection process is adaptive.

# 14. Build a “human-in-the-loop” approval system

This is probably the most credible version.

Don't have AI autonomously authorize major repairs.

Instead:

AI

↓

Detect

↓

Prioritize

↓

Recommend action

↓

Human approves

↓

Automatic dispatch

↓

AI verifies completion

For low-risk maintenance, you could allow automatic work orders.

For major infrastructure:

Engineer approval required.

That gives you a very defensible safety architecture.

# What I would actually build in 3 days

Don't attempt all 14 ideas.

Build one complete automated workflow.

## Your demo:

### 🚗 1. Drive

Raspberry Pi collects:

Camera + IMU + GPS

↓

### 🤖 2. Identify

AI detects:

pothole/crack

↓

### 📍 3. Locate

GPS associates defect with:

Road Segment #43

↓

### 📊 4. Prioritize

AI calculates:

HIGH PRIORITY

because:

- severe defect

- high traffic

- repeated detection

↓

### 🔧 5. Automate inspection

System creates:

Inspection Work Order #1847

↓

### 👷 6. Assign

AI chooses:

Crew A

↓

### 🗺️ 7. Route

System generates:

optimal inspection route

↓

### 🛠️ 8. Maintenance recommendation

Patch pavement

↓

### 📷 9. Verify

Second vehicle pass:

Defect no longer detected.

↓

### ✅ 10. Close

Work order automatically completed.

# Your dashboard could literally have these 4 columns

## IDENTIFY

23 anomalies detected

## PREDICT

7 showing deterioration

## INSPECT

4 inspections automatically scheduled

## ACT

2 crews dispatched

That's a very clean response to the competition prompt.

And the killer differentiator is:

We're not building another infrastructure inspection camera. We're building an autonomous workflow that turns continuously collected physical-world data into maintenance and recovery actions.

For a 3-day competition, I'd spend roughly 30% of your time on the Raspberry Pi/sensor, 40% on the AI + prioritization logic, and 30% on the dashboard/work-order automation. The last two are what will make the prototype feel like a real company rather than a science project.

### For building automated inspection and action workflows, an AI app builder may help.

Replit inc

An AI Assistant for One Task

Tell Replit the goal and get a focused assistant for a useful result.

Ad

More options

Sponsored options

Ad

Sponsored options

Ad

Today 9:58 AM

Hosseini, Amin MOR531_Lean_Canvas_Template (3).xlsx

Spreadsheet

fill out each table with detail, ideas and alternatives

I filled out every Lean Canvas section with detailed ideas, alternatives, positioning options, customer segments, AI/technology differentiation, GTM, metrics, costs, and revenue models.

I also kept the recommendations centered around the strongest concept we discussed: continuous infrastructure intelligence using vehicle-based sensor fusion, while giving you alternatives so you can decide how narrow or ambitious to make the startup.

Download the completed Lean Canvas

Infrastructure_AI_Lean_Canvas_Filled.xlsxSpreadsheetOpen file

would there be a way to adjust this for deep structural issues using sensors and geospatial tech?

Yes. In fact, I think deep structural monitoring could make the concept substantially more differentiated than a standard pothole/road-inspection platform.

The important adjustment is that you don't position the system as a camera that can magically "see through concrete." Instead, build a multi-layer structural health monitoring system that combines inexpensive continuous sensing with targeted geospatial and non-destructive-testing data.

## The revised concept

Instead of:

Detect visible infrastructure damage

move toward:

Continuously identify structural anomalies, track how infrastructure is changing spatially and over time, and automatically direct expensive inspections to the assets most likely to have deeper problems.

Think:

Continuous low-cost monitoring → anomaly detection → geospatial analysis → targeted deep inspection → structural health model

# 1. The sensor stack

You could have several layers.

| Sensor | What it measures | Potential structural insight |
| --- | --- | --- |
| RGB camera | Cracks, spalling, corrosion, deformation | Visible symptoms |
| IMU / accelerometer | Vibration, acceleration, impacts | Abnormal dynamic response |
| GPS/GNSS | Position | Exact asset/location tracking |
| RTK-GNSS | Centimeter-level positioning | Settlement/deformation monitoring |
| LiDAR | 3D geometry | Deflection, deformation, clearance, geometry changes |
| Thermal camera | Temperature patterns | Moisture/delamination indicators |
| Ultrasonic | Internal material response | Cracks, voids, delamination in suitable materials |
| GPR | Subsurface electromagnetic response | Voids, layers, embedded features/anomalies |
| Acoustic sensors | Structural sound/vibration | Potential cracking/material changes |
| Strain sensors | Structural deformation | Load/stress changes |
| Tilt sensors | Angular displacement | Settlement/leaning |
| Satellite InSAR | Surface movement | Millimeter-scale ground/structure displacement over time |

The key is not putting every sensor on every vehicle.

# 2. Create a tiered inspection architecture

This is where I think the idea becomes really compelling.

### Tier 1 — Continuous monitoring

Cheap sensors on existing vehicles:

Camera + IMU + GPS

Your vehicles continuously collect observations.

Cost:

Low

Coverage:

Huge

Purpose:

Find anomalies.

### Tier 2 — Geospatial monitoring

When something looks suspicious, combine it with:

- historical imagery

- LiDAR

- satellite data

- RTK-GPS

- elevation models

- road geometry

- flood zones

- soil/geology

- previous inspection data

Now you're asking:

Is this isolated surface damage or part of a larger structural/geotechnical pattern?

### Tier 3 — Specialized inspection

Only suspicious locations receive expensive inspection:

Thermal → LiDAR → ultrasonic → GPR → drone → engineering inspection

This is important economically.

Instead of:

Inspect everything deeply.

You do:

Continuously screen everything → deeply inspect the anomalies.

# 3. Geospatial technology could be a major differentiator

This is probably the most interesting addition to your current concept.

Imagine you detect a crack at:

Latitude X / Longitude Y

Your system doesn't just create a pin.

It asks:

### What else is happening around this location?

For example:

CRACK DETECTED

↓

GPS / GIS

↓

Historical imagery

↓

Elevation model

↓

Soil/geology

↓

Drainage

↓

Flood history

↓

Satellite deformation

↓

Traffic/loading

↓

Previous inspections

↓

Structural risk model

Now you might discover:

Crack growth + 12 mm surface movement + drainage problem + repeated heavy-vehicle loading.

That's much more meaningful than:

"Crack detected."

# 4. Use geospatial time series

This could become one of your strongest features.

Suppose a bridge approaches a slope or embankment.

Your system tracks:

2026

Elevation change: 0 mm

2027

+4 mm

2028

+9 mm

2029

+17 mm

The system identifies:

Accelerating displacement

That's a very different type of infrastructure intelligence.

You're no longer just doing computer vision.

You're monitoring physical change over time.

# 5. Satellite InSAR is particularly interesting

For certain applications, satellite radar interferometry can measure very small changes in the Earth's surface over time.

That could potentially help identify:

- ground settlement

- subsidence

- landslide movement

- deformation around infrastructure

- structural displacement patterns

You could combine:

Satellite

"This region is moving."

with

Vehicle

"This bridge/road has developing cracks."

with

LiDAR

"Geometry has changed."

with

IMU

"Dynamic response is abnormal."

Now you have multiple independent signals pointing toward the same problem.

# 6. Build a "Structural Anomaly Score"

Instead of pretending the AI can directly diagnose structural failure, create an anomaly/risk score.

For example:

### Bridge 017

| Signal | Observation |
| --- | --- |
| Surface cracking | ↑ 24% |
| Crack width | ↑ 18% |
| LiDAR geometry | 11 mm change |
| Ground movement | 8 mm/year |
| Vibration | 1.7× baseline |
| Thermal anomaly | Detected |
| Traffic loading | High |
| Historical condition | Deteriorating |

### Result

Structural anomaly: HIGH

Recommend targeted engineering inspection

This is much more defensible than:

"Bridge has a 78% chance of failing."

# 7. The AI architecture changes

Your original system was essentially:

Vision AI → defect

I'd evolve it into:

PHYSICAL ASSET

│

┌───────────────┼────────────────┐

↓               ↓                ↓

CAMERA            IMU              GPS

│               │                │

└───────────────┼────────────────┘

↓

SENSOR FUSION

│

┌────────┴────────┐

↓                 ↓

VISUAL DAMAGE      DYNAMIC RESPONSE

│                 │

└────────┬────────┘

↓

GEOINT LAYER

│

┌───────────────┼───────────────┐

↓               ↓               ↓

SATELLITE         LiDAR           GIS

│               │               │

└───────────────┼───────────────┘

↓

TEMPORAL ANALYSIS

↓

STRUCTURAL ANOMALY

↓

PRIORITY ENGINE

↓

┌───────────────┴──────────────┐

↓                              ↓

ROUTINE MAINTENANCE             DEEP INSPECTION

↓

NDT / ENGINEER

That is a much more interesting architecture.

# 8. You can make the system adaptive

This is where the platform could get really smart.

Suppose your system sees:

Bridge A

Minor surface cracking.

But:

Bridge B

Minor surface cracking + increasing vibration + ground movement.

The system shouldn't treat them equally.

It could say:

### Bridge A

Low anomaly

Continue monitoring.

### Bridge B

High anomaly

Deploy:

LiDAR + thermal + ultrasonic inspection.

So the AI isn't just detecting damage.

It is deciding what level of inspection is justified.

# 9. This gives you a strong economic argument

Specialized structural inspections can be expensive.

You don't necessarily want to compete with them.

Instead:

Use inexpensive continuous sensing to determine where expensive inspection resources should be deployed.

That's a much clearer ROI story.

### Current model

Inspect 1,000 assets

↓

Expensive engineering inspection

↓

Find 40 problematic assets

### Your model

Monitor 1,000 assets continuously

↓

AI identifies 80 anomalies

↓

Specialized inspection of 80

↓

Engineers focus on highest-risk assets

The exact economics would need customer validation, but the business thesis is compelling.

# 10. Your moat becomes much stronger

Your original moat:

Computer vision + fleet data

New moat:

Longitudinal multimodal infrastructure dataset

For every asset you could eventually store:

ASSET #38291

Location

│

├── Images

├── Video

├── IMU

├── GPS

├── LiDAR

├── Thermal

├── Satellite history

├── Ground movement

├── Weather

├── Traffic

├── Inspection reports

├── Repairs

└── Structural history

Then you can learn:

What does infrastructure deterioration actually look like before a serious failure?

That dataset could be extremely valuable.

# 11. Where I would focus first

I wouldn't try to detect every type of structural failure for the competition.

I'd choose one asset where the sensor combination makes intuitive sense.

### Option A — Bridges

Probably the strongest demonstration.

Camera
 → cracks/corrosion

IMU
 → vibration

LiDAR
 → geometry

GPS
 → location

Satellite
 → surrounding deformation

AI
 → structural anomaly

### Option B — Roads / pavement

Much easier to prototype.

Camera
 → cracks/potholes

IMU
 → impacts

LiDAR
 → rutting/deformation

GPS
 → location

Satellite
 → subsidence

This is probably the easiest MVP.

### Option C — Utility infrastructure

Potentially very valuable.

Camera
 → poles/lines/vegetation/corrosion

LiDAR
 → geometry/clearance

Thermal
 → overheating

GPS
 → asset location

Satellite
 → terrain movement

This gives you a path into utilities.

# What I would change in your Lean Canvas

Instead of positioning the company as:

AI infrastructure damage detection

I'd change it to:

### GroundSignal

Continuous structural intelligence for the physical world.

Core product:

A multimodal sensing and geospatial platform that continuously monitors infrastructure, identifies structural anomalies, tracks deterioration, and directs specialized inspections to the locations that need them most.

And your core loop becomes:

Sense → Geolocate → Fuse → Detect → Track → Prioritize → Inspect → Learn

That's substantially more differentiated than a standalone pothole-detection system, while still allowing you to start with a very simple camera + IMU + GPS MVP and progressively add LiDAR, thermal, satellite, and NDT capabilities.

what are the biggest infrastructure problems in LA?

Yes. Looking specifically at Los Angeles as a potential test market for your infrastructure-monitoring startup, there are several problems that fit your sensor + geospatial concept particularly well.

The important thing is that LA doesn't just have a "pothole problem." It has a combination of aging infrastructure, deferred maintenance, seismic/geotechnical risk, flooding, wildfire exposure, and fragmented asset management. LA's own 2026 Capital Infrastructure Program explicitly describes past infrastructure planning as fragmented and notes underinvestment in streets, parks, buildings, and facilities.

## The biggest opportunities for your company

| LA problem | What is happening | Fit for your technology |
| --- | --- | --- |
| 🛣️ Road deterioration | Poor pavement, potholes, cracking, rutting, congestion | ⭐⭐⭐⭐⭐ |
| 🌧️ Stormwater / flooding | Aging drainage systems + extreme rainfall | ⭐⭐⭐⭐⭐ |
| 🌎 Earthquake / seismic risk | Structures and utilities exposed to seismic events | ⭐⭐⭐⭐⭐ |
| ⛰️ Landslides / ground movement | Hillsides and transportation infrastructure vulnerable to land movement | ⭐⭐⭐⭐⭐ |
| 🌉 Bridge condition | Aging structures + structural/seismic concerns | ⭐⭐⭐⭐⭐ |
| 🚰 Water infrastructure | Aging pipes, leaks, breaks, system reliability | ⭐⭐⭐⭐ |
| ⚡ Utility infrastructure | Grid, poles, substations, gas infrastructure exposed to hazards | ⭐⭐⭐⭐ |
| 🚇 Transit infrastructure | Aging rail/bus infrastructure and reliability | ⭐⭐⭐⭐ |
| 🚶 Sidewalks / curb ramps | Accessibility, cracking, trip hazards, missing infrastructure | ⭐⭐⭐⭐ |
| 🔥 Wildfire infrastructure | Roads, utilities and other assets exposed to wildfire | ⭐⭐⭐⭐ |

California's 2025 infrastructure assessment gives particularly weak grades to roads (D), stormwater (D), drinking water (D+), bridges (C-), and transit (C-), while identifying aging assets, deferred maintenance and climate hazards as major challenges.

# 1. Roads — easiest starting point

This is still probably your best MVP.

California's 2025 report says nearly one-third of roads are in poor condition and estimates a roughly $70 billion statewide maintenance/rehabilitation funding shortfall over the next decade.

Your system could monitor:

- potholes

- cracks

- rutting

- road deformation

- faded markings

- damaged signs

- debris

- drainage problems

- pavement deterioration

### Why LA is interesting

You could put your sensor on:

garbage truck → bus → utility truck → city vehicle

and have it continuously collect road-condition data.

Instead of:

"Here are 4,000 potholes."

you give Public Works:

"Here are the 50 road defects most likely to become expensive problems."

# 2. Stormwater is potentially an even better fit

This is one I'd seriously consider for your competition.

California's stormwater infrastructure received a D, with much of the drainage infrastructure more than 50 years old. LA County's Measure W is expected to generate about $5.6B over 20 years, while the estimated cost of achieving the relevant water-quality objectives was cited at about $24B.

Your system could detect:

- blocked drains

- damaged catch basins

- standing water

- erosion

- sinkholes

- damaged culverts

- drainage overflow

- road deformation associated with water

- flood damage

Then combine this with:

elevation + rainfall + drainage GIS + satellite + historical observations

to identify areas likely to develop problems.

### That's much more interesting than potholes.

You could say:

"We don't just detect the pothole. We identify the drainage or subsurface condition causing it."

# 3. Ground movement / landslides

This is where your geospatial + structural idea becomes particularly interesting.

LA County's 2025 hazard-mitigation plan specifically identifies land movement as a threat to transportation, bridges, rail, pipelines, electrical infrastructure and gas infrastructure.

Imagine:

Satellite/InSAR
 → detects ground movement

LiDAR
 → identifies terrain change

Vehicle camera
 → detects road cracking

IMU
 → detects abnormal road surface response

GPS
 → precisely locates everything

Then:

"This road isn't simply cracking. It is located in an area experiencing measurable ground movement."

That's a much stronger structural-risk signal.

# 4. Bridges — best use case for your "deep structural" vision

This is probably the most compelling long-term application.

California has more than 25,000 bridges, and ASCE reports that more than 1,500 are in poor condition.

Your system could monitor:

### Visual

- cracking

- corrosion

- spalling

- exposed rebar

- joint deterioration

### Geometry

- LiDAR

- deformation

- displacement

### Dynamics

- accelerometers

- vibration signatures

### Geospatial

- ground movement

- seismic exposure

- flood exposure

- historical deformation

### Temporal

- Is the crack growing?

- Is the bridge geometry changing?

- Is vibration changing?

The output becomes:

Structural anomaly detected → targeted engineering inspection recommended.

That's much more defensible than claiming your AI can directly determine whether a bridge is safe.

# 5. Water pipes / underground infrastructure

This is where your system could eventually become really differentiated.

The camera obviously can't see a buried water pipe.

But you can detect surface manifestations.

For example:

Road depression

↓

Repeated wet pavement

↓

Ground movement

↓

Pipe/GIS location

↓

Historical leak reports

↓

Satellite / geospatial data

↓

Potential underground infrastructure anomaly

Your system could tell the utility:

"Investigate this location."

Then they deploy specialized equipment.

That gives you a very compelling philosophy:

### Don't try to replace expensive inspection equipment.

Use cheap sensors to determine where expensive inspection should happen.

# 6. Utilities

LA's hazard plan specifically identifies land movement risks to electrical infrastructure and gas pipelines.

A utility truck could continuously monitor:

- poles

- transformers

- substations

- vegetation clearance

- exposed infrastructure

- road/ground deformation

- construction damage

- signs of corrosion

- access roads

Then combine the observations with the utility's existing asset GIS.

You could eventually build:

"Infrastructure digital twin + continuous field observations."

# 7. Sidewalks and pedestrian infrastructure

This is less technically impressive, but potentially very easy to demonstrate.

LA's current mobility planning identifies sidewalks/crossings, traffic safety, transit infrastructure and walking/biking infrastructure among major community concerns.

Your vehicle could detect:

- sidewalk cracks

- trip hazards

- missing curb ramps

- damaged curb ramps

- blocked sidewalks

- faded crossings

- damaged signs

- bike-lane obstructions

This is a great early commercial use case because the defects are visually detectable and don't require specialized structural sensors.

# What I'd actually build for LA

I'd combine these into a three-layer LA infrastructure intelligence system:

### Layer 1 — Surface

Camera + AI

Detect:

cracks, potholes, sidewalks, signs, drainage, visible damage

↓

### Layer 2 — Physical

IMU + LiDAR + GPS

Detect:

impacts, deformation, geometry changes, abnormal vibration

↓

### Layer 3 — Geospatial

Satellite + GIS + terrain + historical data

Detect:

subsidence, landslide risk, flood exposure, seismic exposure, infrastructure proximity

↓

### Result

## Infrastructure Anomaly Score

Not:

"This bridge will fail."

Instead:

"This asset exhibits multiple abnormal signals and warrants targeted inspection."

# The killer LA demo

You could actually make your competition demo very LA-specific:

### Example

A vehicle drives down a road.

Camera

Detects growing crack.

IMU

Abnormal vibration.

GPS

Precise location.

LiDAR

Detects slight road deformation.

GIS

Road is near a drainage channel.

Satellite

Area has historical ground movement.

Your system produces:

🔴 HIGH PRIORITY — STRUCTURAL / GEOTECHNICAL ANOMALY

Recommended action:

Deploy targeted LiDAR/thermal/NDT inspection.

That is far more differentiated than "AI finds potholes."

### Your LA beachhead could therefore be:

Roads → drainage → ground movement → bridges → utilities

with the same underlying platform.

The long-term thesis becomes:

Los Angeles has millions of physical assets and thousands of vehicles already moving throughout the city. GroundSignal turns that existing movement into a continuous sensor network for detecting infrastructure deterioration before it becomes an expensive failure.

Kistler Instrumente AG

Live Bridge Monitoring

Explore Kistler's digital Structural Health Monitoring on a real bridge – connect with our team.

Ad

More options

### Chat paused until usage resets at 2:59 PM

You’ve reached the limit for chats that include files or images. Start a new text-only chat or upgrade to continue now.

New chat

Claim free offer

what are the types of materials that LA bridges uses and what tech could be used to detect structural issues? how could we detect seimmic bearing joints strenght from an outside persepctive?

Sponsored options

Ad